Day 1 — From the outside to the product architecture

1. Physical Inspection — Studying the Product

Start with the device without opening it.

Students should document:

- Product name/model- Samsung Galaxy A30s

- Manufacturer- Samsung

- Approximate age/version- 6 Years

- External dimensions- 158.5 x 74.7 x 7.8 mm

- Buttons/switches- 3 buttons + 1 switch

- Mainboard / PCB Components-

| # | Component | Type | Part No. |
| --- | --- | --- | --- |
| 1 | Processor (SoC) | BGA | Exynos 7904 |
| 2 | RAM | BGA (LPDDR4) | K4FBE3D4HM (4 GB) |
| 3 | ROM / Storage | BGA (V-NAND) | KLMCG2UCTA (64 GB) |

- Processor (SoC)-

| Parameter | Detail |
| --- | --- |
| SoC | Samsung Exynos 7 Octa 7904 |
| Process | 14 nm |
| CPU | Octa-core (2× Cortex-A73 @ 1.8 GHz + 6× Cortex-A53 @ 1.6 GHz) |
| Architecture | 64-bit ARMv8.2-A |
| GPU | ARM Mali-G71 MP2 @ 770 MHz |
| Modem | LTE Cat.12 (600 Mbps DL / 150 Mbps UL) |
| Memory controller | LPDDR4X, 2×16-bit bus |
| Storage interface | eMMC 5.1 |
| ISP | Up to 32 MP single / 16+16 MP dual |
| Video | 4K@30fps capture, 1080p@60fps playback |
| Transistor count | ~2 billion |
| TDP | ~5 W |
| Package | BGA (on mainboard) |

- LEDs/displays-

Main Display - 6.4" Super AMOLED panel- Front

Camera Flash- LED (white)- Rear, next to camera module

- Connectors- Total 10 Connectors

| # | Connector | Type | Location |
| --- | --- | --- | --- |
| 1 | Battery connector | FPC | Mainboard |
| 2 | LCD/Display FPC (78-pin) | FPC | Mainboard |
| 3 | Inter-board flex (main ↔ sub) | FPC | Mainboard ↔ Sub-board |
| 4 | Front camera | FPC | Mainboard |
| 5 | Rear camera (triple) | FPC | Mainboard |
| 6 | Coaxial antenna cable | Coax | Mainboard |
| 7 | Charging port (USB-C) | Board-to-board | Sub-board |
| 8 | Fingerprint sensor | FPC | Sub-board |
| 9 | Earpiece speaker | FPC | Frame |
| 10 | Vibration motor | FPC | Frame |

Power source-Battery( 4000 mAh Li-Po, non-removable, model EB_BA505ABU)

- Ports- Total 3 Ports

| # | Port | Type | Location |
| --- | --- | --- | --- |
| 1 | USB Type-C | Data + Charging | Bottom |
| 2 | 3.5 mm Audio Jack | Analog audio | Bottom |
| 3 | SIM + microSD tray | Nano-SIM + microSD (shared slot) | Right side |

- Sensors- 6 Sensors

| # | Sensor | Type | Location |
| --- | --- | --- | --- |
| 1 | Fingerprint | Optical, under-display | Display assembly |
| 2 | Accelerometer | MEMS | Mainboard |
| 3 | Gyroscope | MEMS | Mainboard |
| 4 | Proximity | IR | Top bezel (under notch) |
| 5 | Ambient Light | Photodiode | Top bezel |
| 6 | Compass / Magnetometer | Hall-effect | Mainboard |

- Mechanical interfaces-

| # | Interface / Component | Type | Location |
| --- | --- | --- | --- |
| 1 | Frame screws | Torx/Phillips (PH000) | Perimeter of mid-frame |
| 2 | Display clips | Plastic snap-fit | Top & bottom of frame |
| 3 | Back cover clips | Plastic snap-fit | Perimeter of frame |
| 4 | Display adhesive | Silicone/PU glue | Between display & frame |
| 5 | Battery adhesive | Double-sided tape | Inside frame |
| 6 | Antenna bands | Metal strips | Frame edges |
| 7 | Mid-frame to back cover | Snap-fit + screws | Full perimeter |
| 8 | Exynos 7904 SoC | BGA IC | Mainboard |
| 9 | S537 PMIC (Power Mgmt) | QFN IC | Mainboard |
| 10 | S2MPU06B PMIC | QFN IC | Mainboard |
| 11 | ET9553M Charging IC | QFN IC | Mainboard |
| 12 | Audio Codec IC | QFN IC | Mainboard |
| 13 | K4FBE3D4HM (4 GB LPDDR4 RAM) | BGA IC | Mainboard |
| 14 | KLMCG2UCTA (64 GB V-NAND) | BGA IC | Mainboard |
| 15 | Electrolytic capacitors | Through-hole / SMD | Mainboard (power rail) |
| 16 | Tantalum capacitors | SMD (3216) | Mainboard (decoupling) |
| 17 | MLCC ceramic capacitors | SMD (0402/0603) | Mainboard (distributed) |

- Labels and markings-

| # | Label / Marking | Location |
| --- | --- | --- |
| 1 | IMEI / Serial No. / Model No. sticker | Back panel (lower area) |
| 2 | BIS / CMII ID sticker (regulatory) | Back panel |
| 3 | Battery label (model, capacity, voltage, warnings) | On battery pack |
| 4 | WEEE / Recycling symbol | Back panel or battery |
| 5 | "Made in" country-of-origin marking | Back panel |
| 6 | QR code / barcode sticker | Back panel |
| 7 | Warning / caution text (e.g., "Do not disassemble", Li-ion warnings) | Back panel or battery |
| 8 | Protective film (front, rear, camera, ports) | Applied over glass/lens/openings |

- Visible screws/seams-

- Screws (all Phillips PH000, 1.5 mm driver):

| # | Screws | Location | Purpose |
| --- | --- | --- | --- |
| 1 | 14 screws | Mid-frame perimeter (under back cover) | Secure mid-frame to display assembly |
| 2 | 1 screw | Mainboard | Secure mainboard to frame |
| 3 | 1 screw | Sub-board | Secure sub-board to frame |

Seams (visible parting lines):

| # | Seam | Location |
| --- | --- | --- |
| 1 | Back cover ↔ Mid-frame | Full perimeter of back panel |
| 2 | Mid-frame ↔ Display frame | Top & bottom edges (snap-fit) |
| 3 | SIM tray opening | Right side |
| 4 | USB-C port opening | Bottom |
| 5 | 3.5 mm jack opening | Bottom |
| 6 | Speaker grille | Bottom |

- Ventilation/openings

| # | Opening | Purpose | Location |
| --- | --- | --- | --- |
| 1 | Speaker grille | Audio output | Bottom edge |
| 2 | Microphone hole (primary) | Voice recording | Bottom edge |
| 3 | Microphone hole (secondary) | Noise cancellation | Top edge |

Then ask three questions:

A. What is this product supposed to do?

Example: A power bank → store energy and provide regulated USB power.

B. What does the user expect?

For a power bank:

Portable → safe → rechargeable → sufficient capacity → reliable USB output → indication of battery status.

C. What engineering decisions can we already infer?

For example:

USB-C input → charging circuit → Li-ion battery → protection → boost converter → USB output

You haven't opened the product yet, but you are already forming a functional hypothesis.

That is the essence of reverse engineering.

2. Product Literature Review

Now move from physical observation → historical research.

Don't just search for the current product.

Look for:

Version 1 → Version 2 → Version 3 → current version

For each version, investigate:

| Parameter | Old version | New version |
| --- | --- | --- |
| Processor | ? | ? |
| Battery | ? | ? |
| Display | ? | ? |
| Connectivity | ? | ? |
| Charging | ? | ? |
| Sensors | ? | ? |
| PCB technology | ? | ? |
| Power consumption | ? | ? |
| Safety features | ? | ? |

Then ask:

Why did the manufacturer change it?

That question takes students beyond component identification into engineering reasoning.

For example:

Old: Micro-USB
↓
New: USB-C

Possible reasons:

- better mechanical usability

- higher charging capability

- reversible connector

- market standardization

- improved power-delivery capability

The important point is to distinguish evidence from hypothesis.

3. Product Analysis — Group Discussion

Now divide students into groups.

Give them three questions:

Purpose

What problem is this product solving?

User expectations

What would users consider a good product?

Trade-offs

What compromises did the designer probably make?

For example, consider a smartphone.

A user wants:

- long battery life

- high performance

- thin body

- low weight

- good camera

- low temperature

- low cost

But engineering creates conflicts:

Higher performance → more power → more heat → larger cooling requirement.

Larger battery → longer runtime → increased weight/thickness.

So reverse engineering isn't merely:

"What components are inside?"

It is:

"Why was the product designed this way?"

That is a much more valuable engineering skill.

4. Introduction to Guided Teardown

Only after documentation and research should students open the device.

The sequence should be:

Document → Photograph → Open → Photograph → Identify → Record

Never immediately start removing components.

Tools

Typical teardown kit:

- Precision screwdriver set

- Plastic pry tools

- Spudger

- Tweezers

- ESD wrist strap

- ESD mat

- Magnifying glass/microscope

- Digital camera/phone

- Multimeter

- Containers for screws

- Labels/markers

For electronics, I would strongly recommend introducing ESD discipline before the first teardown.

5. ESD Precautions

Students should understand why ESD matters.

A person can carry a static charge that is not noticeable to them but can damage semiconductor devices.

So establish a simple rule:

Before touching the PCB:

Ground yourself → use ESD protection → handle PCB by its edges → avoid touching IC pins unnecessarily.

Also teach students that power must be removed before disassembly, and batteries/capacitors may retain hazardous energy.

For mains-powered equipment, this becomes a significantly more serious safety issue; students should not casually dismantle energized or high-voltage equipment.

Day 2 — Guided Teardown

Now the real reverse engineering begins.

Step 1 — Opening the device

The instructor should demonstrate first.

Students record:

- Screw locations

- Connector locations

- Cable routing

- Mechanical clips

- PCB mounting points

- Shields

- Battery location

- Antenna location

- Heat sinks

- Grounding points

Very important:

Photograph before removing anything.

Then photograph again after each major layer is removed.

This creates a disassembly record.

6. Internal Hardware Analysis — Part I

Now create a component inventory.

Don't ask students merely:

"What is this component?"

Instead ask them to build a structured table.

| Ref./Location | Component | Package | Marking | Function | Notes |
| --- | --- | --- | --- | --- | --- |
| U1 | MCU | QFN | XYZ123 | Main controller | ? |
| U2 | Regulator | SOT-23 | ABC | 3.3 V supply | ? |
| C1 | Capacitor | 0603 | — | Decoupling | Near U1 |
| R1 | Resistor | 0603 | 10K | Pull-up | I²C |
| J1 | Connector | USB-C | — | Power/data | — |

This transforms the teardown into engineering data collection.

Day 3 — Component Markings, Ratings & Specifications

This is where students move from identification to datasheet-based reverse engineering.

Suppose they find an IC marked:

STM32F103C8T6

They should determine:

- Manufacturer

- Part number

- MCU family

- CPU architecture

- Flash

- RAM

- Operating voltage

- Package

- GPIO count

- Communication peripherals

- ADC

- Maximum clock frequency

Then connect the datasheet information back to the physical PCB.

7. Component Functional Analysis

This is probably the most important activity in the whole module.

Don't stop at:

"U1 is an STM32."

Ask:

"What role does U1 play in this product?"

Create a Function Table.

| Component | Electrical role | Functional role | Connected to |
| --- | --- | --- | --- |
| MCU | Digital processing | Product controller | Sensors, display |
| LDO | Voltage regulation | Creates 3.3 V rail | Battery → MCU |
| MOSFET | Electronic switching | Power/load control | MCU + load |
| Crystal | Clock source | MCU timing | MCU |
| EEPROM | Non-volatile memory | Stores configuration | MCU |
| Sensor | Signal generation | Measures physical parameter | MCU |

Then build the larger picture.

Example architecture

┌──────────────┐

│   POWER      │

│ Battery/Input│

└──────┬───────┘

│

┌──────▼───────┐

│ Power Mgmt   │

│ LDO/DC-DC    │

└──────┬───────┘

│

┌────────────┼─────────────┐

│            │             │

▼            ▼             ▼

Sensors       MCU          Memory

│            │

│      ┌─────┼─────┐

│      │     │     │

▼      ▼     ▼     ▼

Display Communication

│

▼

External

Device

Now the students are no longer looking at a PCB as a collection of components.

They are seeing it as a system architecture.

The key skill I would emphasize

Your module should progressively develop these five levels of thinking:

Level 1 — Observation

"I see a USB-C connector, several ICs and a battery."

↓

Level 2 — Identification

"This IC is an STM32Fxxx."

↓

Level 3 — Specification

"The datasheet says it operates at 3.3 V and contains these peripherals."

↓

Level 4 — Functional understanding

"This MCU reads the sensor and controls the display."

↓

Level 5 — Engineering reasoning

"The designer probably selected this MCU because it provides the required peripherals, processing capability and power characteristics at the target cost."

That Level 5 is the real objective of reverse engineering.

Recommended student deliverables

By the end of these three days, every team should produce:

- External Product Inspection Sheet

- Product Version/Literature Comparison

- User Needs & Trade-off Analysis

- Teardown Photo Log

- Mechanical Disassembly Sequence

- Component Inventory/BOM

- Component Marking & Datasheet Table

- Functional Block Diagram

- Component Function Table

- Final Reverse-Engineering Report

And I would add one more deliverable:

"Reverse Engineering Conclusions"

Students should answer:

What did we learn about the designer's decisions that was not obvious from simply looking at the product?